Cadence Tools Command Reference

# Table of Contents

Getting Started on Linux for Cadence Lab

Simulation Tool (ncverilog)

Checking Code Coverage in ncverilog using IMC (Incisive Metrics Center)

Synthesis Tool (genus)

Syntax for writing the Constraints file (.sdc)

Equivalence Checking (conformal LEC)

Linting

Static Timing Analysis (Tempus)

Additional References

# Getting Started on Linux for Cadence Lab

> create a folder in the desktop, with your srn/name

> open the folder

> right-click and create files for design and testbench, 
eg. db_fsm.v and db_tb.v

> right-click on the files and open them using gedit, save the design and testbench codes in the respective files

> right-click inside the folder and select open in terminal

> enter the following commands in the terminal

## csh

Enters the C-Shell

## source /home/<install location>/cshrc

Navigates to the Cadence Tools install path and starts the tool

Note: You can use the upper arrow in the terminal to navigate quickly to the already used paths/commands and use tab-key to auto-complete commands.

> A new window appears that welcomes the user to the Cadence Design Suite, the following tools can be invoked in this window.

# Simulation Tool (ncverilog)

>To start reading the design and testbench files, to obtain a waveform in the Graphical User Interface (simvision),  enter the following commands. 
Note: No space between +access and +rw, but mandatory space between +rw and +gui. (make sure to follow all similar spacing patterns given in the tool reference)

## ncverilog <design> <testbench> +access+rw +gui

eg. ncverilog db_fsm.v  db_tb.v  +access+rw  +gui

Note: the +gui starts up the ncverilog GUI window.

> navigate through the design hierarchy and select the signals you want to analyze in the design browser (hold down ctrl-key while selecting), right-click and select send to waveform

> in the simvision window, select the play button, followed by the pause button to start and stop the simulation. The simulation will end automatically if the $finish statement is executed in the HDL.

> select the ‘=’ symbol at the top right corner of the window, to fit the waveform’s entirety in the same frame.
> drag the red marker to the beginning of the waveform and select on the ‘+’ symbol on the top right corner, to magnify until the waveform pulses are visible for verifying the functionality of the design.

# Checking Code Coverage in ncverilog using IMC  (Incisive Metrics Center)

## ncverilog design.v tb.v +access+rw +gui +nccoverage+all

> Check for the path of the file “cov_work” generated in the terminal then type:

(Invoke Incisive Metrics Center)

>enter the command ‘imc’ in the terminal which will launch the IMC GUI.

## imc

> In he IMC’s Graphical User Interface, you can navigate and select the file to check the Code Coverage (block, branch, expression, toggle) and FSM Coverage, represented in percentages.

# Synthesis Tool (genus)

## genus -gui

Opens the genus tool with gui, alternatively you can show and hide gui using command gui_show and gui_hide

## read_libs <path of .lib file>

Reads library file for synthesis, from the specified path. Eg. saed90nm_typ.lib the 90 nanometer typical library

## read_hdl <path of design file>

Reads design file to be synthesized, written in HDL (eg. verilog, systemverilog)

## elaborate

Elaborates the design in the tool, and can be viewed in the GUI by selecting Hier Cell > Schematic View(Module) > in New.

## *For Synthesis with constraints*

## read_sdc <path of .sdc constraints file>

## syn_generic

Synthesizes the design to the G Tech cells (default cells for the Cadence Tool)

## syn_map

maps the synthesized cells to the library specified earlier in read_libs command

## syn_opt -incremental

Incrementally optimizes the synthesized design

## report_timing > (path for .rpt file to save timing report)

Reports timing Time Borrowed, Uncertainty, Required Time, Launch Clock, Data Path and Slack.

## report_area > (path for .rpt file to save area report)

Reports area of the synthesized design in micro-meters-square

## report_power > (path for .rpt file to save power report)

Reports power in nano Watts (nW)

## write_hdl > (path for .v file for netlist to be written)

Writes the netlist in HDL format in the path specified

## quit

Exits the genus tool

# Syntax for writing the Constraints file (.sdc)

## Creating Clock “clk”

set EXTCLK "clk" ; set_units -time <units eg.1.0ns> ; set EXTCLK_PERIOD <time period eg. 20.0>; create_clock -name "$EXTCLK" -period "$EXTCLK_PERIOD" -waveform "0 [expr $EXTCLK_PERIOD/2]" [get_ports clk]

## Setting clock skew

set SKEW <skew value eg. 0.200> set_clock_uncertainty $SKEW [get_clocks $EXTCLK]

## Setting source rise and fall latency

set SRLATENCY <source rise latency eg. 0.80> set SFLATENCY <source fall latency eg. 0.75>

## Setting Rise times and Fall Times

set MINRISE <minimum rise time eg. 0.20> set MAXRISE <maximum rise time eg. 0.25> set MINFALL <minimum fall time eg. 0.15> set MAXFALL <maximum fall time eg. 0.10> set_clock_transition -rise -min $MINRISE [get_clocks $EXTCLK] set_clock_transition -rise -max $MAXRISE [get_clocks $EXTCLK] set_clock_transition -fall -min $MINFALL [get_clocks $EXTCLK] set_clock_transition -fall -max $MAXFALL [get_clocks $EXTCLK]

## Setting Input and Output Delays for ports in the design

set INPUT_DELAY <input delay value eg. 0.5> set OUTPUT_DELAY <output delay value eg. 0.5> set_input_delay -clock [get_clocks $EXTCLK] -add_delay 0.3 [get_ports <input port name>] set_output_delay -clock [get_clocks $EXTCLK] -add_delay 0.3 [get_ports <output port name>]

Note: the port names must correlate with the names used in the modules, and can be validated by using the report_timing command while synthesizing with constraints in the Cadence Genus tool

# Equivalence Checking (conformal LEC)

## Opening the Cadence Conformal Logical Equivalence Checking tool

lec -LPGXL

Note: The GUI can be used alternative to entering the commands

## Reading Library

GUI: file -> Read library -> choose the library’s .v file Command Line: read library -Both -Replace -sensitive -Verilog technology_file.v -nooptimize

## Reading Verilog file (GOLDEN)

GUI: file -> Read design for RTL(Verilog code) -Golden -> choose the RTL verilog code Command Line: read design design.v -Verilog -Golden -sensitive -continuousassignment Bidirectional -nokeep_unreach -nosupply

## Reading Netlist file (REVISED)

GUI: file -> Read design for RTL netlist or other code we want to compare with -Revised Command Line: read design design2_or_netlist.v -Verilog -Revised -sensitive -continuousassignment Bidirectional -nokeep_unreach -nosupply

## Set mode:

set system mode lec

## Add points at which equivalence should be checked(Here we check all points):

add compared points -all

## Run the comparison:

compare

The Equivalent and Non-Equivalent points are displayed in the GUI window, and can Non Equivalent modules/ sub-modules can be viewed by clicking on the ‘Non-Equivalent’ label

# Linting (irun)

## To run linting on the design file

irun -superLint <path of the HDL file>

The rtlchecks.log and modelchecks.log files are generated in your present working directory.

The tool uses automated structural analysis on the design so that it complies with design coding rules that prevent synthesis issues & functional bugs and enforce coding styles for readability & re-use. These rtlchecks.log and modelchecks.log files present the *N - notes, *W - warnings and *E - errors for the design.

# Static Timing Analysis (Tempus)

## Invoke the Tempus tool and open GUI

tempus

## Alternatively: Invoke Tempus without GUI

tempus -no_gui

## Read library used for synthesis:

read_lib library_file.lib

## Read synthesized gate level netlist:

read_verilog netlist.v

## Set the top module in your design:

set_top_module top

## Read the constraints file:

read_sdc constraints.sdc

## Setting things to check: Signal integrity check

set_delay_cal_mode -siAware true set_si_mode -enable_delay_report true set_si_mode -enable_glitch_report true set_si_mode -enable_glitch_propagation true

## Update the settings to report all timing parameters

update_timing -full

## Report timing: (general report with parameters like slack and critical path)

report_timing

## Report slack alone:

report_slack

report_timing -path_type summary_slack_only

## Report clocks:

report_clocks

## Analysis report generation:

report_analysis_coverage

# Additional References

Changing power engine from joules to legacy in Genus Tool

set_db power_engine legacy

Sending files from one system to another in cadence lab

scp <filename> <sender’s username>@<ipaddress of reciever>:<~ or receiving path > Eg. scp /home/cmos/Desktop/DSD_lab.zip @10.3.32.69:</home/cmos/Desktop>

Incisive Formal Verifier(IFV):

IFV tool can either read scripts(f file) or directly read from .sv file. Optionally a bind file can be created to link separate design(.sv) file and assertions file(.sva).

Run basic:

ifv example.sv

Will display in the command prompt if the assertions failed or passed.

ifv example.sv +gui

-> Click on the run button on the top right corner of the tool bar.

-> Right click on “Pass” or “Failed” to look for traces where it passed or failed.

Will open simvision to display the traces.

exit